Jarvis Local Voice Assistant

Build Documentation

Replacing Amazon Alexa with a fully local, private voice assistant

Date: June 18, 2026

Author: Joe Gitta

Status: Phase 4 Complete — ThinkStation satellite fully operational

# 1. Project Overview

The goal of this project is to replace all Amazon Alexa devices in the home with a fully local, privacy-first voice assistant called "Jarvis." The system runs entirely on-premises with no cloud dependencies for voice processing, using Home Assistant as the hub and Ollama for optional LLM queries.

Key privacy and functionality goals:

- No audio ever leaves the local network

- Wake word detection runs locally (hey_jarvis model)

- Speech-to-text runs locally (OpenAI Whisper via faster-whisper)

- Text-to-speech runs locally (Piper / Amy voice)

- Home automation commands processed by Home Assistant natively

- Optional LLM queries via Ollama (llama3.2:3b on local GPU)

## 1.1 Architecture Summary

| Wake Word Engine | wyoming-openwakeword on ThinkStation (port 10400 localhost) |
| --- | --- |
| Voice Satellite | wyoming-satellite on ThinkStation (port 10700, Wyoming protocol) |
| STT (Speech-to-Text) | Whisper add-on in Home Assistant (faster-whisper, port 10300) |
| TTS (Text-to-Speech) | Piper add-on in Home Assistant (port 10200, Amy/low voice) |
| Conversation Agent | Home Assistant built-in (device control & intents) |
| LLM (optional) | Ollama llama3.2:3b on ThinkStation (port 11434) |
| Home Automation Hub | Home Assistant on Proxmox VM 106 |
| Device Controller | Hubitat (Z-Wave/Zigbee) integrated into HA |

# 2. Infrastructure & Network

## 2.1 Home Assistant Server

| External URL | https://ha.jgitta.com |
| --- | --- |
| Internal IP:Port | 192.168.88.39:8123 |
| Proxmox VM ID | 106 (node: jg-hud) |
| VM Name | home-assistant |
| RAM | 10 GB |
| HA Version | 2026.6.3 (Home Assistant OS) |

## 2.2 ThinkStation-P710 (Voice Satellite + Ollama Server)

| Hostname | ThinkStation-P710 |
| --- | --- |
| IP Address | 192.168.88.41 |
| SSH Alias | thinkstation |
| OS | Ubuntu 22.04 LTS |
| CPU | Intel Xeon (multi-core) |
| GPU | NVIDIA GTX 1050 Ti (4GB VRAM — used by Ollama) |
| Audio Input | MXL USB Microphone (Burr-Brown PCM2900B, ALSA card 3) |
| Audio Output | Analog speakers via PCH/ALC662 (ALSA card 0) |
| Audio Server | PipeWire (locks hardware; aplay must use -D default) |

# 3. Implementation Phases

## Phase 1 — Home Assistant Voice Pipeline Add-ons

Installed three HA add-ons that form the core STT/TTS/wake-word pipeline. All configured with boot: auto and set to running.

| Add-on | Version / Port / Role |
| --- | --- |
| core_whisper | 3.2.0 — STT — core-whisper.local.hass.io:10300 |
| core_piper | 2.2.2 — TTS — core-piper.local.hass.io:10200 |
| core_openwakeword | 2.1.0 — Wake Word — core-openwakeword.local.hass.io:10400 |

Wyoming integrations added in HA Settings > Devices & Services:

- openWakeWord  →  Entry ID: 01KVEV21ZHYGZBWHV40KTSE739

- Piper  →  Entry ID: 01KVEVWNJ1CJWHT2314BNBVTY6

- Whisper  →  Entry ID: 01KVEVX27X6YFYCK9RE5HDQ0N1

## Phase 2 — Ollama LLM on ThinkStation

Ollama was installed on the ThinkStation to provide optional LLM conversation capability. The service was configured to bind to all interfaces so HA can reach it over the LAN.

- Ollama installed via official installer script

- Model pulled: llama3.2:3b (~2GB, fits in GTX 1050 Ti 4GB VRAM)

- Service override created to bind to 0.0.0.0 (see config files section)

- Integrated into HA via Ollama integration (Entry ID: 01KVEW7EF5NYVHJ2NQ9PVBBHZ4)

- URL: http://192.168.88.41:11434

IMPORTANT NOTE — Ollama not used for device control:

Ollama (llama3.2:3b) outputs raw JSON function-call syntax instead of natural language when handling HA device intents. The Jarvis pipeline uses the Home Assistant built-in conversation agent for all device control. Ollama is available as an alternate pipeline for general LLM queries.

## Phase 3 — Jarvis Pipeline in Home Assistant

Created the "Jarvis" voice assistant pipeline in HA Settings > Voice Assistants.

| Pipeline Name | Jarvis |
| --- | --- |
| Language | English |
| STT Engine | faster-whisper |
| STT Language | English |
| Conversation Agent | Home Assistant (built-in — handles device control) |
| TTS Engine | Piper |
| TTS Language | en_US |
| TTS Voice | amy (low) |
| Wake Word | hey_jarvis (configured per satellite) |

## Phase 4 — ThinkStation Wyoming Satellite

Configured the ThinkStation-P710 as a voice satellite using the Wyoming protocol. The satellite listens for the wake word locally, then streams audio to HA for STT processing and plays back TTS responses.

- wyoming-openwakeword installed at /opt/wyoming-openwakeword/.venv

- wyoming-satellite installed at /opt/wyoming-satellite/.venv

- Separate Python venvs required (version conflict: wyoming==1.5.4 vs >=1.8)

- Wyoming integration added in HA pointing to 192.168.88.41:10700

- Satellite assigned to: Area=Lab, Pipeline=Jarvis, Voice=amy (low)

Audio device resolution:

  - Mic: MXL USB (Burr-Brown PCM2900B)  →  ALSA CARD=CODEC,DEV=0

  - Speaker: Analog out  →  aplay -D default (routes through PipeWire)

  - PipeWire holds /dev/snd hardware exclusively; plughw:CARD=PCH fails with "Host is down"

  - Fix: use aplay -D default + add XDG_RUNTIME_DIR=/run/user/1000 to systemd service

# 4. Software Installed on ThinkStation

## 4.1 System Packages

- python3-venv, python3-pip, python3-dev

- build-essential, git

- alsa-utils (arecord, aplay, amixer, alsactl)

- pipewire, wireplumber (audio server — pre-installed on Ubuntu 22.04)

## 4.2 Python Packages (two separate venvs)

### wyoming-openwakeword venv  (/opt/wyoming-openwakeword/.venv)

- setuptools

- wyoming-openwakeword (from GitHub: rhasspy/wyoming-openwakeword)

- wyoming >= 1.8 (installed as dependency)

### wyoming-satellite venv  (/opt/wyoming-satellite/.venv)

- setuptools

- wyoming-satellite (from GitHub: rhasspy/wyoming-satellite, installed without [all] extras)

- wyoming == 1.5.4 (installed as dependency — incompatible with openwakeword venv)

- NOTE: installed without [all] extras to avoid numpy/Python 3.12 build failure

## 4.3 Ollama

- Installed via: curl -fsSL https://ollama.com/install.sh | sh

- Model: llama3.2:3b

- Service: /etc/systemd/system/ollama.service (auto-created by installer)

- Override: /etc/systemd/system/ollama.service.d/override.conf

# 5. Configuration Files

## 5.1  /etc/systemd/system/ollama.service.d/override.conf

Makes Ollama listen on all network interfaces instead of localhost only, so HA can reach it from 192.168.88.39.

[Service]

Environment="OLLAMA_HOST=0.0.0.0"

## 5.2  /etc/systemd/system/wyoming-openwakeword.service

Runs the wake word engine locally on the ThinkStation, listening on localhost only (satellite connects to it via TCP).

[Unit]

Description=Wyoming openWakeWord

After=network.target

[Service]

Type=simple

User=jgitta

ExecStart=/opt/wyoming-openwakeword/.venv/bin/wyoming-openwakeword \

--uri tcp://127.0.0.1:10400 \

--model hey_jarvis

Restart=always

RestartSec=5

[Install]

WantedBy=multi-user.target

## 5.3  /etc/systemd/system/wyoming-satellite.service

Runs the Wyoming satellite. Listens on port 10700 for HA connections. Uses MXL USB mic for capture and routes TTS audio through PipeWire to analog speakers.

[Unit]

Description=Wyoming Satellite

After=network.target wyoming-openwakeword.service

Wants=wyoming-openwakeword.service

[Service]

Type=simple

User=jgitta

Environment="XDG_RUNTIME_DIR=/run/user/1000"

ExecStart=/opt/wyoming-satellite/.venv/bin/wyoming-satellite \

--name "ThinkStation" \

--uri tcp://0.0.0.0:10700 \

--mic-command "arecord -D plughw:CARD=CODEC,DEV=0 -r 16000 -c 1 -f S16_LE -t raw -q" \

--mic-command-rate 16000 \

--mic-command-width 2 \

--mic-command-channels 1 \

--snd-command "aplay -D default -r 22050 -c 1 -f S16_LE -t raw -q" \

--snd-command-rate 22050 \

--snd-command-width 2 \

--snd-command-channels 1 \

--snd-volume-multiplier 3.0 \

--wake-uri tcp://127.0.0.1:10400 \

--wake-word-name hey_jarvis \

--awake-wav /opt/wyoming-satellite/sounds/awake.wav \

--done-wav /opt/wyoming-satellite/sounds/done.wav \

--debug

Restart=always

RestartSec=5

[Install]

WantedBy=multi-user.target

## 5.4  ALSA Volume State  (/var/lib/alsa/asound.state)

Saved via 'sudo alsactl store' to persist volume settings across reboots.

- Headphone output: 100% unmuted (required — was muted by default)

- Speaker output: 100%

- Master: 94%

- MXL USB (CODEC card): PCM playback 70%

# 6. Home Assistant Integration Reference

## 6.1 Wyoming Integration Entry IDs

These IDs are used when calling HA's config entry APIs directly.

| Integration | Entry ID |
| --- | --- |
| Wyoming — openWakeWord | 01KVEV21ZHYGZBWHV40KTSE739 |
| Wyoming — Piper (TTS) | 01KVEVWNJ1CJWHT2314BNBVTY6 |
| Wyoming — Whisper (STT) | 01KVEVX27X6YFYCK9RE5HDQ0N1 |
| Ollama | 01KVEW7EF5NYVHJ2NQ9PVBBHZ4 |

## 6.2 HA API Notes

- HA version 2026.x changed the subentry flow API handler format

- Handler MUST be a JSON array: {"handler": ["<entry_id>", "<sub_type>"]}

- Sending handler as a plain string causes a 500 ValueError

- Subentry endpoint: POST /api/config/config_entries/subentries/flow

## 6.3 Satellite Device Settings (HA)

| Device Name | ThinkStation |
| --- | --- |
| Area | Lab |
| Assigned Pipeline | Jarvis |
| Voice | amy (low) |
| Wyoming Host | 192.168.88.41 |
| Wyoming Port | 10700 |

## 6.4 Key Integrations Active in HA

- Hubitat (8b841069) — Z-Wave/Zigbee devices via Hubitat hub

- Whisper — STT add-on (local)

- Piper — TTS add-on (local)

- openWakeWord — wake word detection add-on (local)

- Ollama — LLM inference on ThinkStation (local, not used in Jarvis pipeline)

- Alexa Media Player — legacy, being phased out

- Frigate — NVR on ThinkStation (192.168.88.41:5000)

# 7. Audio Device Reference (ThinkStation)

## 7.1 ALSA Card Map

| Card | Name / Chip / Role |
| --- | --- |
| Card 0 — PCH | HDA Intel PCH / ALC662 rev3 — Analog speaker output |
| Card 1 — Webcam | BY-5640 USB Webcam — Built-in USB mic (not used) |
| Card 2 — NVidia | HDA NVidia — HDMI audio outputs (not used) |
| Card 3 — CODEC | Burr-Brown PCM2900B — MXL USB Microphone (mic + headphone) |

## 7.2 PipeWire Conflict Resolution

PipeWire (the system audio server) holds /dev/snd hardware exclusively. Direct hardware access via plughw: fails with 'Host is down' or 'Device or resource busy' from within systemd services.

Solution applied:

- Speaker output: use aplay -D default (routes via PipeWire ALSA plugin)

- Mic input: use arecord -D plughw:CARD=CODEC,DEV=0 (MXL is USB, separate from PCH)

- Add Environment="XDG_RUNTIME_DIR=/run/user/1000" to the systemd service

- This allows aplay -D default to find the PipeWire socket at /run/user/1000/pipewire-0

PipeWire default sink: alsa_output.pci-0000_00_1b.0.analog-stereo (PCH analog output)

# 8. Voice Command Notes

## 8.1 Entity Name Matching

The Home Assistant built-in conversation agent matches voice commands to entity names exactly (case-insensitive). Plural/singular matters.

- Say "stairwell light" not "stairwell lights" — entity is light.stairwell_light

- Say "backroom light" not "backroom lights"

- View all 122 exposed entities: HA Settings > Voice Assistants > Expose tab

- Add aliases to an entity to allow multiple spoken names (click entity > Voice Assistants > add alias)

## 8.2 Tested Commands

- "Hey Jarvis, what time is it?" — works

- "Hey Jarvis, turn on the stairwell light" — works (controls Hubitat Z-Wave device)

- "Hey Jarvis, turn off the stairwell light" — works

- "Hey Jarvis, what is the weather in Waterloo, Iowa" — LLM responds (general query)

# 9. Known Issues & Fixes

## 9.1 Issues Encountered and Resolved

| Issue | Fix Applied |
| --- | --- |
| wyoming-satellite needs wyoming==1.5.4; openwakeword needs >=1.8 (conflict) | Separate Python venvs at /opt/wyoming-satellite/.venv and /opt/wyoming-openwakeword/.venv |
| numpy build fails on Python 3.12 with [all] extras | Install wyoming-satellite without [all] extras: pip install -e . |
| setuptools not found during pip install | Run: pip install setuptools before installing package |
| HA subentry flow API returns 500 ValueError | Send handler as JSON array: ["entry_id", "sub_type"] not as string |
| wyoming-openwakeword flag --wake-word is invalid | Correct flag is --model hey_jarvis |
| aplay fails with 'Device or resource busy' (PipeWire) | Use aplay -D default instead of plughw:CARD=PCH |
| aplay fails with 'Host is down' from systemd service | Add Environment="XDG_RUNTIME_DIR=/run/user/1000" to [Service] section |
| Headphone output muted by default (no audio heard) | Run: amixer -c 0 set Headphone 87 unmute && alsactl store |
| Ollama outputs raw JSON for device commands | Use Home Assistant built-in conversation agent; Ollama only for general LLM queries |
| mDNS auto-discovery creates duplicate Wyoming entries in HA | Ignore all three duplicate discoveries in HA UI |

# 10. Service Management Commands

## 10.1 Wyoming Services (run on ThinkStation via SSH)

# Check status

systemctl status wyoming-satellite

systemctl status wyoming-openwakeword

# View live logs (watch for 'Detection(name=hey_jarvis)')

journalctl -u wyoming-satellite -f

journalctl -u wyoming-openwakeword -f

# Restart both services

sudo systemctl restart wyoming-openwakeword

sudo systemctl restart wyoming-satellite

# After editing service files

sudo systemctl daemon-reload

sudo systemctl restart wyoming-satellite

# Save ALSA volume settings (persist across reboot)

sudo alsactl store

## 10.2 Ollama Service

# Check status

systemctl status ollama

# Pull a different/larger model

ollama pull llama3.2:3b

ollama pull llama3.1:8b

# List downloaded models

ollama list

## 10.3 Diagnose Wake Word / Pipeline Issues

# Check if wake word fired and what STT heard:

journalctl -u wyoming-satellite --since '5 minutes ago' | grep -E 'Detection|transcript|synthesize|error'

# Check audio devices available:

arecord -l    # capture devices

aplay -l      # playback devices

# Test mic recording (stop satellite first):

sudo systemctl stop wyoming-satellite

arecord -D plughw:CARD=CODEC,DEV=0 -r 16000 -c 1 -f S16_LE -d 4 /tmp/test.wav

# Test speaker playback:

aplay -D default /opt/wyoming-satellite/sounds/awake.wav

# Check what processes are holding audio devices:

fuser /dev/snd/*

# 11. Next Steps / Future Work

- Add room satellites — HA Voice PE (~$59) or ESP32-S3-BOX-3 (~$50) per room

  - HA Voice PE: plug-and-play, sold at home-assistant.io/voice

  - ESP32-S3-BOX-3: flash via ESPHome, no Linux setup needed

- Phase out Alexa Echo devices room by room as satellites are deployed

- Tune Ollama system prompt for Jarvis personality and home context

- Upgrade to llama3.1:8b or llama3.2:8b model for better general LLM responses

- Add aliases to entity names to allow natural speech variations (e.g., 'stairwell lights' -> 'stairwell light')

- Consider extended_openai_conversation HACS integration for LLM + device control in one agent